PART A

EACH QUESTION CARRIES 2 MARKFor all mechanical click here

UNIT-I

1. Define stress.

When an external force acts on a body, it undergoes deformation. At the same time the body resists deformation. The magnitude of the resisting force is numerically equal to the applied force. This internal resisting force per unit area is called stress.

Stress = Force/Area

When a body is subjected to an external force, there is some change of dimension in the body. Numerically the strain is equal to the ratio of change in length to the original length of the body.= P/A unit is N/mm^2

2. Define strain

δ Strain = Change in length/Original length e = δL/L

3. State Hooke’s law.

It states that when a material is loaded, within its elastic limit, the stress is directly proportional to the strain.

Stress α Strain

σ α e

Where,

σ = Ee

E = σ/e unit is N/mm^2

E - Young’s modulus

σ - Stress e - Strain

4. Define shear stress and shear strain.

The two equal and opposite force act tangentially on any cross sectional plane of the body tending to slide one part of the body over the other part. The stress induced is called shear stress and the corresponding strain is known as shear strain.

5. Define Poisson’s ratio.

When a body is stressed, within its elastic limit, the ratio of lateral strain to the longitudinal strain is constant for a given material.

Poisson’ ratio (µ or 1/m) = Lateral strain /Longitudinal strain

6. State the relationship between Young’s Modulus and Modulus of Rigidity.

E = 2G (1+1/m)

Where,

E - Young’s Modulus

K - Bulk Modulus

1/m - Poisson’s ratio

7. Define strain energy

Whenever a body is strained, some amount of energy is absorbed in the body. The energy which is absorbed in the body due to straining effect is known as strain energy.

8. Give the relationship between Bulk Modulus and Young’s Modulus.

E = 3K (1-2/m)

Where,

E - Young’s Modulus

K - Bulk Modulus

1/m - Poisson’s ratio

9. What is compound bar?

A composite bar composed of two or more different materials joined together such that system is elongated or compressed in a single unit.

10. Define- elastic limit

Some external force is acting on the body, the body tends to deformation. If the force is released from the body its regain to the original position. This is called elastic limit

11. Define – Young’s modulus

The ratio of stress and strain is constant with in the elastic limit.

E = Stress

Strain

12. Define Bulk-modulus

The ratio of direct stress to volumetric strain.

K = Direct stress

Volumetric strain

13. Define- lateral strain

A direct stress is always accompanied by a strain its own direction and the opposite kinds of strain in every direction at right angles to it. Such a strain is known as secondary strain or lateral strain.

13. Define- longitudinal strain

The strain right angle to the direction of the applied load is called lateral strain.

14. Define- Rigidity modulus

The shear stress is directly proportional to shear strain.

N = Shear stress

Shear strain

15. What are the types of elastic constants?

There are three types of elastic constants. They are:-

a) Young's Modulus (E), b) Bulk Modulus (K) and   c) Shear Modulus(C)

16. Define strain energy.

The energy absorbed in a body, when strained within its elastic limit is known as strain energy. It is also known as resilience.

17. Define proof resilience.

The maximum strain energy that can be stored in a body is known as proof resilience.

18. Define modulus of resilience.

The proof resilience per unit volume is known as modulus of resilience. It is also known as strain energy density.

PART B

EACH QUESTION CARRIES 12 MARKS

- Draw a typical stress-strain curve for a mild steel rod subjected to a tensile test and indicate the salient points on the curve.

A typical tensile test curve for the mild steel has been shown below

Nominal stress – Strain OR Conventional Stress – Strain diagrams:

Stresses are usually computed on the basis of the original area of the specimen; such stresses are often referred to as conventional or nominal stresses.

True stress – Strain Diagram:

Since when a material is subjected to a uniaxial load, some contraction or expansion always takes place. Thus, dividing the applied force by the corresponding actual area of the specimen at the same instant gives the so called true stress.

SALIENT POINTS OF THE GRAPH:

  - So it is evident from the graph that the strain is proportional to strain or elongation is proportional to the load giving a st. line relationship. This law of proportionality is valid upto a point A.

Or we can say that point A is some ultimate point when the linear nature of the graph ceases or there is a deviation from the linear nature. This point is known as the limit of proportionality or the proportionality limit.

  - For a short period beyond the point A, the material may still be elastic in the sense that the deformations are completely recovered when the load is  removed.  The limiting point  B  is  termed as Elastic Limit.

  - and (D) - Beyond the elastic limit plastic deformation occurs and strains are not totally recoverable. There will be thus permanent deformation or permanent set when load is removed. These two points are termed as upper and lower yield points respectively. The stress at the yield point is called the yield strength.

A study a stress – strain diagrams shows that the yield point is so near the proportional limit that for most purpose the two may be taken as one. However, it is much easier to locate the former. For material which do not possess a well define yield points, In order to find the yield point or yield strength, an offset method is applied.

In this method a line is drawn parallel to the straight line portion of initial stress diagram by off setting this by an amount equal to 0.2% of the strain as shown as below and this happens especially for the low carbon steel.

- A further increase in the load will cause marked deformation in the whole volume of the metal. The maximum load which the specimen can with stand without failure is called the load at the ultimate strength.

The highest point ‘E' of the diagram corresponds to the ultimate strength of a material.

σu = Stress which the specimen can with stand without failure & is known as Ultimate Strength or  Tensile Strength.

σu is equal to load at E divided by the original cross-sectional area of the bar.

- Beyond point E, the bar begins to forms neck. The load falling from the maximum until fracture occurs at F.

[Beyond point E, the cross-sectional area of the specimen begins to reduce rapidly over a relatively small length of bar and the bar is said to form a neck. This necking takes place whilst the load reduces, and fracture of the bar finally occurs at point F]

Note: Owing to large reduction in area produced by the necking process the actual stress at fracture is often greater than the above value. Since the designers are interested in maximum loads which can be carried by the complete cross section, hence the stress at fracture is seldom of any practical value.

- Derive the relationship between the elastic constants.

Relation among elastic constants

Relation between E, G and µ :

Let us establish a relation among the elastic constants E,G and  . Consider a cube of material of side  ‘a' subjected to the action of the shear and complementary shear stresses as shown in the figure and producing the strained shape as shown in the figure below.

Assuming that the strains are small and the angle A C B may be taken as 450.

Therefore strain on the diagonal OA

= Change in length / original length

Since angle between OA and OB is very small hence OA OB therefore BC, is the change in the  length of the diagonal OA

Now this shear stress system is equivalent or can be replaced by a system of direct stresses at 450 as shown below. One set will be compressive, the other tensile, and both will be equal in value to the applied shear strain.

Thus, for the direct state of stress system which applies along the diagonals:

We have introduced a total of four elastic constants, i.e E, G, K and µ. It turns out that not all of these are independent of the others. Infact given any two of then, the other two can be found.

irrespective of the stresses i.e, the material is incompressible.

When µ = 0.5 Value of k is infinite, rather than a zero value of E and volumetric strain is zero, or in other words, the material is incompressible.

Relation between E, K an G:

Consider a cube subjected to three equal stresses	as shown in the figure below

The total strain in one direction or along one edge due to the application of hydrostatic stress or volumetric stress	is given as

Relation between E, G and K :

3. Abar of mild steel has an overall length of 2.1 m. The diameter up to 700 mm

length is 56 mm, the diameter of the remaining 1.4 m is 35 mm. Calculate the extension of the bar due to a tensile load of 55 kN.

E = 200 GN/m2.

4. A steel bar of 20 mm diameter and 400 mm long is placed concentrically inside a gunmetal tube. The tube has inside diameter 22 mm and thickness 4 mm. The length of the tube exceeds the length of the steel bar by 0.12 mm. Rigid plates are placed on the compound assembly. Find: a) the load which will just make tube and bar of same length and b) the stresses in the steel and gunmetal when a load of 50 kN is applied. E for steel=213 GN/m2, E for gunmetal=100 GN/m2.

SOLUTION:

5. Two steel rods and one copper rod each of 20 mm diameter together support a

load of 50 kN as shown in Fig. 2.22. Find the stress in each rod. Take Es = 200 GPa,Eb = 100 GPa

UNIT-II

PART A

EACH QUESTION CARRIES 2 MARK

1. What are the types of beams?

1. Cantilever beam

2. Simply supported beam

3. Fixed beam

4. Continuous beam

5. over hanging beam

2. What are the types of loads?

1. Concentrated load or point load

2. Uniform distributed load

3. Uniform varying load

3. What is mean by transverse loading on beam?

If a load is acting on the beam which perpendicular to the central line of it then it is called transverse loading.

4. What is Cantilever beam?

A beam one end free and the other end is fixed is called cantilever beam.

5. What is simply supported beam?

A beam supported or resting free on the support at its both ends.

6. What is mean by over hanging beam?

If one or both of the end portions are extended beyond the support then it is called over hanging beam.

7. What is mean by concentrated loads?

A load which is acting at a point is called point load.

8. What is uniformly distributed load.

If a load which is spread over a beam in such a manner that rate of loading ‘w’ is uniform through out the length then it is called as udl.

9. Define point of contra flexure? In which beam it occurs?

Point at which BM changes to zero is point of contra flexure. It occurs in overhanging beam.

10. What is mean by positive or sagging BM?

BM is said to positive if moment on left side of beam is clockwise or right side of the beam is counter clockwise.

11. What is mean by negative or hogging BM?

BM is said to negative if moment on left side of beam is counterclockwise or right side of the beam is clockwise.

12. Define shear force and bending moment?

SF at any cross section is defined as algebraic sum of all the forces acting either side of beam.

BM at any cross section is defined as algebraic sum of the moments of all the forces which are placed either side from that point.

13. When will bending moment is maximum?

BM will be maximum when shear force change its sign.

14. What is maximum bending moment in a simply supported beam of span ‘L’

subjected to UDL of ‘w’ over entire span?

Max BM =wL2/8

15. In a simply supported beam how will you locate point of maximum bending moment?

The bending moment is max. When SF is zero. Write SF equation at that point

and equating to zero we can find out the distances ‘x’ from one end .then find maximum bending moment at that point by taking all moment on right or left hand side of beam.

16. What is shear force?

The algebric sum of the vertical forces at any section of the beam to the left or right of the section is called shear force.

17. What is shear force and bending moment diagram?

It shows the variation of the shear force and bending moment along the length of the beam.

PART B

EACH QUESTION CARRIES 10 MARKS

- A cantilever beam is subjected to various loads as shown in below figure. Draw the shear force diagram and bending moment diagram for the beam.

Fig: Shear force and bending moment

Solution: Consider a section (X – X’) at a distance x from section B. shear force. between B and D;

Shear force Fx = + wx

At x = 0, Fb = 0 …(1)

x = 1 m; Fd just right = 2 × 1 = 2 kN

S.F. between D and C; Fx = + wx + 5

At x = 1 m; FD just left = (2 × 1) + 5 = 7 kN

At x = 1.5 m; Fc just right = (2 × 1.5) + 5 = 8 kN

S.F. between C and A

Fx = + wx + 5 + 4

At x = 1.5 m; Fc just left = 2 × 1.5 + 5 + 4 = 12 kN

At x = 2 m; FA = 2 × 2 + 5 + 4 = 13 kN

Bending moment between C and A;

Mx = – (wx).x/2

Mx = wx2/2 – 5 (x – 1) – 4 (x – 1.5)

At x = 1.5 m; Mc = –2 × (1.5)2 / 2 –5 (1.5 – 1) – 4 (1.5 – 1.5)

Mc = –4.75 kN- m

x = 2.0 m; Ma = – 2 × (2)2 / 2 – 5 (2.0 – 1) – 4 (2.0 – 1.5)

Ma = - 11 kN-m

(The sign of bending moment is taken to be negative because the load creates hogging).

The shape of bending moment diagram is parabolic in shape from B to D, D to C, and, also C to A.

- A simply supported beam overhanging on one side is subjected to a uniform distributed load of 1 kN/m. Sketch the shear force and bending moment diagrams and find the position of point of contra-flexure.

Consider a section (X – X’) at a distance x from end C of the beam.

To draw the shear force diagram and bending moment diagram we need RA and RB.

Fig: simply supported beam carrying –UDL

By taking moment of all the forces about point A.

RB × 3 – w/2 × (4)2 = 0

RB = 1 × (4)2 / 2 × 3 = 8/3 kN

For static equilibrium;

RA + RB – 1 × 4 = 0

RA = 4 – 8/3 = 4/3 kN

To draw shear force diagram we need shear force at all salient points:

Taking a section between C and B, SF at a distance x from end C. we have,

At x = 0; FC = 0

x = 1 m; FB just right = 1 × 1 = + 1 kN

Now, taking section between B and A, at a distance x from end C, the SF is:

At x = 4 m; FA = 4 – 8/3 = + 4/3 kN = + 1.33 kN

The shear force becomes zero;

═ x = 2.67 m

(The sign is taken positive taken when the resultant force is in downward direction the RHS of the section).

To draw bending moment diagram we need bending moment at all salient points. Taking section between C and B, bending moment at a distance x from end C, we have

Mx = – wx2 / 2 = –1.x2/2 kN m

When x = 0, MC = 0

At x = 1 m. MB = –1 × (1)2 / 2 = –0.5 kN m

Taking section between B and A, at a distance x from C, the bending moment is:

Mx = –x2/2 + 8/3 (x – 1)

At x = 1 m, MB = – 0.5 kN m

x = 4 m; MA = –(4)2 /2 – 8/3 (4 – 1) = 0

The maximum bending moment occurs at a point where

dMx / dx = 0

═ d/dx [–x2 / 2 + 8/3 x – 8/3] = 0

═ –1/2 × 2x + 8/3 = 0

═ x = 8/3 m from end C.

═ Mmax = –1/2 (8/3)2 + 8/3 (8/3 – 1) = 0.89 kN m

The point of contraflexure occurs at a point, where

Mx = 0

═ –x2/2 + 8/3 (x – 1) = 0

═ x2 = 16/3 (x – 1)

═ x2 – 16/3 x + 16/3 = 0

x = 1.335 m or 4 m

- A simply supported beam is subjected to a combination of loads as shown in figure. Sketch the shear force and bending moment diagrams and find the position and magnitude of maximum bending moment.

Solution: To draw the shear force diagram and bending moment diagram we need RA and RB.

Fig. Shear force and bending moment

By taking moment of all the forces about point A.

We get RB × 10 – 8 × 9 – 2 × 4 × 5 – 4 × 2 = 0

RB = 12 kN

From condition of static equilibrium ΣFy = 0

RA + RB – 4 – 8 – 8 = 0

RA = 20 – 12 = 8 kN

To draw shear force diagram we need shear force at all salient points:

For AC; FA = + RA = 8 kN

For CD, FC = + 8 – 4 = 4 kN

FD = 4 kN

For DE, Fx = 8 – 4 – 2 (x – 3) = 10 – 2x

At x = 3 m; FD = 10 – 6 = 4 kN

At x = 7 m; FE = 10 – 2 × 7 = –4 kN

The position for zero SF can be obtained by 10 – 2x = 0

x = 5 m

For EF; Fx = 8 – 4 – 8 = – 4 kN

For FB; Fx = 8 – 4 – 8 – 8 = – 12 kN

To draw BMD, we need BM at all salient points.

For region AC, Mx = + 8x

At x = 0; MA = 0

x = 2; MC = 8 × 2 = 16 kN m

For region CD; Mx = + 8x – 4 (x – 2)

At x = 2 m; MC = 8 × 2 – 4 (2 – 2) = 16 kN m

At x = 3 m; MD = 8 × 3 – 4 (3 – 2) = 20 kN m

For region DE,

Mx = + 8x – 4 (x – 2) – 2(x – 3)2 / 2 = 10x – x2 – 1

At x = 3 m; MD = 8 × 3 – 4 (3 – 2) – 2(32 – 3)2 / 2 = 20 kN m

At x = 7 m; ME = 10 × 7 – (7)2 – 1 = 20 kN m

At x = 5 m; MG = 10 × 5 – (5)2 – 1 = 24 kN m

For region EF,

Mx = 8x – 4 (x – 2) – 2 × 4 (x – 5) = 48 – 4x

At x = 9 m, MF = 120 – 12 × 9 = 12 KN m

At x = 10 m; MB = 120 – 12 × 10 = 0

The shear force diagram and bending moment diagram can now be drawn by using the various values of shear force and bending moment. For bending moment diagram the bending moment is proportional to x, so it depends, linearly on x and the lines drawn are straight lines.

The maximum bending moment exists at the point where the shear force is zero, and also dM/dx = 0 in the region of DE

d/dx (10 x – x2 – 1) = 0

10 – 2x = 0

X = 5 m

Mmax = 10 × 5 – (5)2 – 1 = 24 kN m

Thus, the maximum bending is 24 kN m at a distance of 5 m from end A.

- A cantilever beam carries a uniform distributed load of 60 kN/m as shown in figure. Draw the shear force and bending moment diagrams for the beam.

Fig.  Simply supported beam carrying UDL upto some extent

Solution: Consider a section (X – X’) at a distance x from end B.

Shear force = Total unbalanced vertical force on either side of the section.

S.F. between B and C Fx = + w.x …(1)

A x = 0, FB = w.0 = 0

x= 1.8 m; FC = 60 × 1.8 = 108 kN

Since, there is no load between points A and C; for this region Fx remains constant. Therefore, from C to A;

Fx = + 108 kN

(The sign is taken to be positive because the resultant force is in downward direction on right hand side of the section).

SFD will be triangular from B to C and a rectangle from C to A.

Now,

Bending moment between B and C Mx = – (wx).x/2 = –wx2/2

At x = 0; MB = –w.0/2 = 0

x = 1.8 m; MC = –60/2.(1.8)2 = –97.2 kN m

For region C to A; Mx = – w (1.8)(x – 1.8/2) = – 60 × 1.8 (x – 0.9) = –108 (x – 0.9)

At x = 1.8 m; MC = –108 (1.8 – 0.9) = –97.2 kN m

x = 2.5 m; MA = –108 (2.5 – 0.9) = –172.8 kN m

(The sign of bending moment is taken to be negative because the load creates hogging).

BMD is parabolic in nature from B to C and straight line from C to A.

- Draw the S.F. and BM. diagrams for the overhanging beam carrying uniformly distributed load of 2 kN/m over the entire length and a point load of 2 kN as shown in Fig. Locate the point of contra-flexure.

First calculate the reactions RA and RB. Upward forces = Downward forces RA + RB = 2 X 6 +2 = 14 kN Taking moments of all forces about A, we get RB x 4 = 2 x 6 x 3 + 2 x 6 = 48 kNm RB = 12 kN ; RA = 2 kN Shear force diagram

| Shear force At point | Shear force towards |  |
| --- | --- | --- |
|  | Left of the section | Right of the section |
| A | -- | 2x6 + 2 – 12 = 2kN |
| B | 2 – 2 x 4 = -6 kN | 2 x 2 +2 = 6 kN |
| C | 2 + 12 – 2 x 6 = 2 kN | -- |

Bending Moment Diagram:

MA = 0

MB = -(2 x 2) x 1 – 2 x 2 = -8kN-m

MC = 0

To find maximum bending moment:

WKT, bending moment is maximum where shear force is zero. Therefore, FD =  0 = RA  -2 x x ;	x=1m

MD = 2 x 1 – 2 x 1 x 0.5 = 1 kN-m

Point of Contra-flexure

This point is at E between A and B, where B.M. is zero after changing its sign. The distance of E from A is obtained by putting Mx = 0 ,in the following equation

UNIT-III

PART A

EACH QUESTION CARRIES 2 MARK

1. Write the assumption in the theory of simple bending?

1. The material of the beam is homogeneous and isotropic.

2. The beam material is stressed within the elastic limit and thus obey hooke’s law.

3. The transverse section which was plane before bending remains plains after bending also.

4. Each layer of the beam is free to expand or contract independently about the layer, above or below.

5. The value of E is the same in both compression and tension.

2. Write the theory of simple bending equation?

M/ I = F/Y = E/R

M - Maximum bending moment

I - Moment of inertia

F - Maximum stress induced

Y - Distance from the neutral axis

E - Young’s modulus

R - Constant.

3. What are the different sections in which the shear stress distribution is to be obtained?

• Rectangular section

• Circular section

• I- section

• T- section

• Miscellaneous section

4. What do you mean by shear stress in beams?

The stress produced in a beam, which is subjected to shear forces is know as

stresses.

5. What is the shear stress distribution rectangular section?

The shear stress distribution rectangular section is parabolic and is given by q = F/2I [d2 /4 – y2]

d = Depth of the beam

y = Distance of the fiber from NA

6. What is the shear stress distribution Circular section?

q = F/3I [R2-y2]

7. State the main assumptions while deriving the general formula for shear stresses

The material is homogeneous, isotropic and elastic

The modulus of elasticity in tension and compression are same. The shear stress is constant along the beam width

The presence of shear stress does not affect the distribution of bending stress.

8. Define: Shear stress distribution

The variation of shear stress along the depth of the beam is called shear stress distribution

9. What is the shear stress distribution value of Flange portion of the I-section?

q= f/2I * (D2/4 - y) D-depth

y- Distance from neutral axis

10. What is the value of maximum of minimum shear stress in a rectangular cross section?

Qmax=3/2 * F/ (bd)

11. What is the shear stress distribution for I-section?

The shear stress distribution I-section is parabolic, but at the junction of web and flange, the shear stress changes abruptly. It changes from F/8I [D2 –d2] to B/b x F/8I [D2

–d2] where D = over all depth of the section d = Depth of the web

b = Thickness of web

B = Over all width of the section.

PART B

EACH QUESTION CARRIES 10 MARKS

QUESTION 1

- A cantilever beam of length 2m fails when a load of 2KN is applied at the free end. If the section is 40mmx60mm, find the stress at the failure.

Solution:

Step 1: Data:

Length of beam = 2m or 2000mm

load at failure = 2KN

Section dimensions = 40mm X 60mm

Step 2: Calculation of moment of inertia

I = bd3/12

= (40) (603)/12

= 7.2X105 mm4

Step 3: Calculation of bending moment about fixed end

M = WL

= (2)(2)

= 4KN-m

Step 4:

Calculation of bending stress

M /I= σ / y

Substitute for above (where y = depth /2= 60/2 = 30mm)

There fore

σ = 166.67N/mm2

- A rectangular beam 200mm deep and 300mm wide is simply supported over the span of 8m. What uniformly distributed load per metre the beam may carry, if the bending stress is not exceed 120N/mm2.

Solution:

Step 1: Data:

Length of beam = 8m or 8000mm

Section dimensions = 300mm X 200mm

maximum bending stress = σ = 120N/mm2.

condition: uniformly distributed load for simply supported beam

Step 2: Calculation of bending moment for the above condition

M = wL2/8

= w (8)2/8

= 8wX106

step 3: Calculation of moment of inertia

I = bd3/12

= (300) (2003) /12

= 2X108 mm4

Step 4:

Calculation of Udl

M /I= σ / y

Substitute for above (where y = depth /2= 200/2 =100mm)

8wX106 /2X108= 120 / 100

w =3X104 N/m or 30 N/mm

- A beam is simply supported and carries a uniformly distributed load of 40KN/m run over the whole span. The section of the beam is rectangular having depth as 500mm.If the maximum stress in the material of the beam is 120N/mm2and moment of inertia of the section is 7x108mm4, find the span of the beam.

Solution:

Step 1: Data:

Depth of beam = 500mm

maximum bending stress = σ = 120N/mm2.

moment of inertia =7x108mm4

Step 2: Calculation of bending moment for the above condition

M = wL2/8

= 40(L)2/8

= 5L2

Step 3: Calculation of length of beam

M /I= σ / y

Substitute for above (where y = depth /2= 500/2 =250mm)

5L2 /7x108= 120 / 250

L=8197.56 mm

- A simply supported beam of length L carries a concentrated load P as shown figure below. Find: (a) The maximum shear stress, the shear flow qj, and the shear stress Tj in the joint between the flange and the web; (b) the maximum bending stress. Given: P = 5 kN and L = 4 m.

Figure . (a) Load diagram and beam cross section; (b) shear diagram; (c) moment diagram

Solution The distance  from the Z axis to the centroid is obtained as follows (Fig. a):

The moment of inertia I about the NA is determined by the parallel axis theorem:

The shear and moment diagrams (Figs. b and c) are sketched by applying the method of sections.

- The maximum shearing stress in the beam takes place at the NA on the cross section supporting the largest shear force V. Consequently, QNA = 50(20)25 = 25×103 mm3. The shear force equals 2.5 kN on all cross sections of the beam (Fig. b), Thus,

The first moment of the area of the flange about the NA is

Shear flow and shear stress in the joint are

- and τj = qj/b = 44.1/0.02 = 2.205 MPa.

- The largest moment takes place at midspan (Fig. c). Equation (5.39) is therefore

UNIT-IV

PART A

(EACH QUESTION CARRIES 2 MARK)

1) What do you mean by principal plane?

ANS: The planes which have no shear stress are known as principal planes.

2) What is the use of Mohr's circle?

ANS: It is used to find out the normal, tangential, resultant and principal stresses and their planes.

3) What is the radius of Mohr's circle?

ANS: Radius of Mohr's circle is equal to the maximum shear stress.

4) What are the planes along which the greatest shear stresses occurs?

ANS: Greatest shear stress occurs at the planes which is inclined at 45? to its normal.

5) What is mean by position of principal planes?

ANS: The planes on which shear stress is zero are known as principal planes. The position of principal is obtained by equating the tangential stress to zero.

6) What is another name of maximum principle stress theory?

ANS: Rakine's theory

7) The maximum tangential stress σt = (σx sin 2θ)/2 is maximum if, θ is equal to?

ANS: 45o

8) What is another name of maximum shear stress theory?

ANS: Guests theory

9)  In Mohr's circle method, compressive direct stress is represented on?

ANS: Negative x-axis

10) Which theory gives satisfactory results for brittle materials?

ANS: Maximum principle stress theory

11) The angle between normal stress and tangential stress is known as angle of?

ANS: Obliquity

12) Which theory gives satisfactory results for ductile materials?

ANS: Maximum shear stress theory

13)  Which of the following formulae is used to calculate tangential stress, when a member is subjected to stress in mutually perpendicular axis and accompanied by a shear stress?

ANS: [(σx – σy)/2 ]sin θ – τ cos 2θ

14) When a component is subjected to axial stress the normal stress is?

ANS: σn=σxCos2θ

15) What do you mean by theories failure?

ANS: Theories of failure are those theories which help us to determine the safe dimensions of a machine component when it is subjected to combined stresses due to various loads acting on it during its functionality.

PART B (EACH QUESTION CARRIES 12 MARKS)

UNIT-IV

- A M.S. shaft having yield stress as 232 MPa is subjected to the following stresses.

σx = 120 MPa, σy= – 60 MPa and ıxy = 36 MPa. Find the factor of safety using:

    - Rankine’s theory of failure,

    - Guest’s theory of failure

5) A SAE 1045 steel rod (oys = 309.9 MPa) of 80 mm diameter is subjected to a bending moment of 3 kNm and torque T. Taking Factor of safety as 2.5, find the maximum value of torque T that can be safely carried by rod according to:

(i) Maximum normal stress  theory

(ii) Maximum shear stress theory

UNIT-V

PART A (EACH QUESTION CARRIES 1 MARK)

1) List out the modes of failure in thin cylindrical shell due to an internal pressure?

ANS: Circumferential or hoop stress and Longitudinal stress

2) What is torsion equation

ANS:

3) Define the shaft?

ANS: The shafts are the machine elements which are used to transmit power in machines.

4) What is formula of polar moment of inertia of hollow shaft?

ANS: J=Π(D4-d4)/32

5) What is formula of polar moment of inertia of solid shaft?

ANS: J=ΠD4/32

6) What is the equation of torque transmitted by shaft?

ANS:

7) What is power transmitted by shaft?

ANS: If T is the applied Torque and ω is the angular velocity of the shaft

P=T ω

8) What is the formula of longitudinal stress?

ANS: σt =Pd/4t

9) Define the angle of twist of shaft?

ANS: If a shaft of length L is subjected to a constant twisting moment T along its length, than the angle through which one end of the bar will twist relative to the other is known is the angle of twist.

10) Define the torsional stiffness?

ANS: The tensional stiffness k is defined as the torque per radius twist

11) Define the Hoop stress?

Ans: This is the stress which is set up in resisting the bursting effect of the applied pressure and can be most conveniently treated by considering the equilibrium of the cylinder.

12) Define the torsion?

ANS: Consider a shaft rigidly clamped at one end and twisted at the other end by a torque T = F.d applied in a plane perpendicular to the axis of the bar such a shaft is said to be in torsion

13) What is formula for maximum bending stress in shafts?

ANS: σmax=32M/ ΠD3

14) Define the modulus of elasticity in shear?

ANS: The ratio of the shear stress to the shear strain is called the modulus of elasticity in shear OR Modulus of Rigidity

15) What is the formula of circumferential stress?

ANS: σC =Pd/2t

PART B (EACH QUESTION CARRIES 12 MARKS)

UNIT-V

1)

2)

3)

4)

5)

Note: tap on below picture👇